## Bash-Style One-Liners & Loops

### 1) List all OUT ports

python verilog_parse.py -y "ls ports --dir out"

### 2) Get fan-in cone (combinational) for one OUT port

python verilog_parse.py -y "fanin OUT_READY --depth 8"

### 3) Find sequential instances (e.g., flops) in a cone

Use --seq so traversal reports/stops at seq boundaries.

python verilog_parse.py -y "fanin OUT_READY --seq" | grep -i flop

### 4) Extract just the instance names from a fan-in report

(assuming last column or a predictable column holds instance)

python verilog_parse.py -y "fanin OUT_READY --seq" | awk '{print $2}'

### 5) FOR LOOP: iterate all output ports, fan-in each, extract flop instance(s), collect unique

for p in $(python verilog_parse.py -y "ls ports --dir out" | awk '{print $1}'); do

echo "=== $p ==="

python verilog_parse.py -y "fanin $p --seq" | awk '/flop|mioc_flop|DFF|FF/ {print $1,$2,$3}'

done | sort -u

- Swap the AWK pattern to match your exact flop naming (e.g., mioc_flop).

- If column positions differ, adjust {print $1,$2,$3} accordingly.

### 6) All OUT ports → report path existence from inputs

for p in $(python verilog_parse.py -y "ls ports --dir out" | awk '{print $1}'); do

python verilog_parse.py -y "paths --from IN_CLK --to $p --max 1" | sed "s/^/$p: /"

done

### 7) Generate JSON export & open GUI

python verilog_parse.py -y "export json --to build/graph.json"

python verilog_parse.py -y "gui --json build/graph.json"

## Troubleshooting Patterns

- No graph loaded: Run --manifest first or read verilog <manifest>.

- Object not found: Verify exact name via find <regex> or ls first.

- Too much output: Use --limit, pipe to head, or grep/awk filters.

- Sequential boundaries: Add --seq to fanin|fanout|paths to surface flops/latches.